Timothy Leishman								Word Count: 00,000

Pocatello, Idaho, 83202

leistimo@isu.edu

Breadboard Power Supply

Timothy Leishman

Senior Instructor, Robotics Engineering Technology

Idaho State University

Figure 1. Breadboard Power Supply Schematic

Introduction

Figure 2. Electronic Components

The electronic circuit design process involves defining the problem, determining a solution, circuit calculations & schematic development, acquiring the needed components, circuit testing (often referred to as breadboarding), PCB or printed circuit board development, PCB assembly, and final testing.  Typically, the breadboarding stage is done using a bench top or Laboratory DC Power Supply. Once the breadboarding phase is complete and the circuit is functionally operational, we often need to replace the laboratory DC power supply with a dedicated power source. A power source can be as simple as a 9V battery or may include purchasing a dedicated high-power rack mount Power Supply subsystem. Often, we can design our own power supplies using basic components rather than buying off the shelf units.

Figure 3. Printed Circuit Boards (PCBs)

While the power supply may not be the most exciting aspect of your electronic projects it is a crucial and foundational component. Poor power supply design can lead to intermittent circuit problems, premature circuit failures, and worst of all frustration. Understanding how power supplies function will make you a better electronic troubleshooter. The Breadboard Power Supply will give you the ability to safely build and test circuits. This small project will also help you develop your soldering skills and expose you to the circuit and PCB development process. Learning how to safely design and build dedicated power supplies will bring life to your current and future electronic projects.

A Problem named Covid

In addition to the pandemic, Covid created an instant demand for online at home learning. Suddenly electronics students everywhere lost access to Laboratory Bench Test Equipment and the direct supervision of a Lab Instructor. Modestly priced oscilloscopes, function generators, and laboratory DC power supplies exist and I encourage advanced students to acquire these tools as they can. However, the introductory student will not likely have access to these type of tools at home and for them the Breadboard PS along with a DMM will provide a, affordable and safe, at home circuit prototyping opportunity.

RMS & Peak Voltage

The wall outlet at your house is 120-volt rms. 120vrms is dangerous. The protection circuit breaker which acts like a safety switch, turning off the voltage, has the maximum current rating of 15 amps to 20 amps. Death can occur from as low as 100mA or 0.1A. The circuit breaker at your house is not designed to save your life from electrical shock, it is designed to prevent the wiring in your home from overheating heating and starting a fire. Do not mess 120v wall power it can kill you!

Figure 4. 120vrms waveform on oscilloscope

Wall voltage known as AC (Alternating Current) voltage. Most of what we do in electronics is DC (Direct Current). Examples of DC devices include: your computer, cell phone, Arduino, Raspberry Pi, and all things powered by batteries. The motors in your RC car are DC. However, some motors are designed to operate using single phase AC or industrial three phase AC voltage. Because our wall power is AC and our electronic circuits are mostly DC we will need to convert the alternating wall current to direct current. The process of converting AC to DC is called Rectification.

Figure 4 shows a captured image of 120vRMS wall AC. Observe that the Oscilloscope displays the image in alternating peak to peak voltage not RMS voltage. To convert peak to peak voltage to RMS the following formula can be used; Vrms= Vp2 OR Vp0.707 . Vp is peak voltage and is equal to Vpp (peak to peak voltage) divided by two (Vp=Vpp2 ). An Oscilloscope displays voltage along the Y axis and time along the X axis. In figure 5 note that channel 1 is set to 50 volts per division and have approximately six major divisions and two half divisions equaling 7 major divisions peak to peak. Now if we multiply 7div×50v/div we get a peak to peak voltage of 350vpp. Dividing 350vpp by 2 gives us a peak voltage of 175vp. Dividing 175vp by 2  gives an RMS voltage of 123.74vRMS≈120vRMS . Peak voltage is important to understand when we are using an oscilloscope and calculating rectification circuits. RMS is used for determining AC power. For example, the power calculations for an incandescent light bulb, audio amplifier, and ac motor would be calculated by multiplying the RMS voltage by the RMS current.

Voltage Rectification

Figure 5. Basic unregulated half-wave rectification

Voltage rectification is the process of changing AC (alternating current) to DC (direct current). Knowing that 100mA’s of current can be lethal and that the general skin resistance of a human is between 10KΩ & 1KΩ (when wet), we can use Ohm’s Law, where Current=VoltageResistance , to determine a relatively safe voltage. If we convert 120vrms to peak we get vp=120V×2 or vp=169.71vp.  Now assuming we were able to rectify the 169.71volts we now use the human resistance of 1KΩ to determine the current potential, i amps=169.71v1000Ω, i=169.71mA. This current is potentially lethal to a human. If 100mA is our threshold for lethal current we can calculate the max voltage by Vmax=1KΩ×100mA, Vmax=100v. Now we know that 100v and above are lethal voltages for humans and less than 100v should be less than lethal. Figure 5 shows a basic unregulated half-wave voltage rectifier, transformer T1 is used for isolation and safety. Notice that T1 has a 4:1 turns ratio. T1 is a step-down transformer meaning that the primary side (pins 1 & 2) have four

Figure 6. transformer secondary voltage waveform

times more turns than the secondary side (pins 3 & 4). A step-down transformers secondary voltage will be divided by the turns’ ratio. For this example, vSecondary=Vprimary4=169.71vp4=42.43vp. Figure 6 shows the secondary voltage waveform from T1 pin 3 to pin 4.  The voltage has been stepped down to around 42vp. The waveform however, is still AC. Notice the frequency for both fig 4 and fig 6. Frequency is measured in hertz (Hz) you will see that both the primary and the secondary are measuring at 60Hz which is the standard frequency for AC power in the United States.

Figure 7. diode anode and cathode

The diode D1 will achieve rectification by acting like a switch and allowing current to flow in only one direction.  When the anode is positive with respect to the cathode, current will flow. When the anode is negative with respect to the cathode the diode will act like an open switch and no current will flow. The output waveform will now appear to have the negative peaks clipped off (See fig. 8). The current is flowing in one direction and basic half-wave rectification has been achieved.  Notice the

Figure 8. half-wave rectification

waveform frequency is still 60Hz for half-wave rectification.

Capacitive Filtering

By adding a filtering capacitor, we can smooth the ripple waveform to make it more like a smooth DC. Figure 9 has added a filter capacitor C1 and load resistor R1. The capacitor will charge immediately when the diode D1 is forward biased. When the diode becomes reverse biased the capacitor will discharge through the load R1. With a large enough capacitance value and load resistor the output will be a smooth DC. We can test this by using a 10uF capacitor with no load, a 10KΩ load, and a 1KΩ load to show the varying degrees of ripple (see figures 10, 11, & 12).

Figure 9. basic unregulated rectifier with filtering

Calculating DC Voltage

When there is ripple present on the waveform, VDC can be thought of as the average waveform voltage. VDC can be calculated by adding the maximum positive peak voltage to the minimum peak voltage and then dividing the sum by two.

Example Calculate VDC for figure 12:

VDC=Vmax+Vmin2

Vmax=2.1 div×20vdiv=42v

Vmin=0.5 div ×20vdiv=10v. Now find VDC.

VDC1KΩ=Vmax+Vmin2= 42v+10v2=26V.

Un-Regulated

Observe that as the load is increased (Ohm’s law I=VR, current or “load” increases as resistance is decreased), the voltage ripple waveform increases and as the ripple increases the effective DC voltage is decreased. This is an example of a rectifier circuit that is Un-Regulated. An un-regulated power supply’s DC voltage will fall as the circuit load current is increased.

Figure 12. voltage output waveform,

10uF filter with 1K ohm load:

VDC1KΩ≈26V

Figure 10. voltage output waveform,

10uf filter with no load resistor:

VDC∞Ω≈42V

Figure 11. voltage output waveform,

10uF filter with 10K ohm load:

VDC10KΩ≈40V