Timothy Leishman								Word Count: 00,000

Pocatello, Idaho, 83202

leistimo@isu.edu

Breadboard Power Supply

Timothy Leishman

Senior Instructor, Robotics Engineering Technology

Idaho State University

Figure 1. Breadboard Power Supply Schematic

Introduction

Figure 2. Electronic Components

The electronic circuit design process involves defining the problem, determining a solution, circuit calculations & schematic development, acquiring the needed components, circuit testing (often referred to as breadboarding), PCB or printed circuit board development, PCB assembly, and final testing.  Typically, the breadboarding stage is done using a bench top or Laboratory DC Power Supply. Once the breadboarding phase is complete and the circuit is functionally operational, we often need to replace the laboratory DC power supply with a dedicated power source. A power source can be as simple as a 9V battery or may include purchasing a dedicated high-power rack mount Power Supply subsystem. Often, we can design our own power supplies using basic components rather than buying off the shelf units.

Figure 3. Printed Circuit Boards (PCBs)

While the power supply may not be the most exciting aspect of your electronic projects it is a crucial and foundational component. Poor power supply design can lead to intermittent circuit problems, premature circuit failures, and worst of all frustration. Understanding how power supplies function will make you a better electronic troubleshooter. The Breadboard Power Supply will give you the ability to safely build and test circuits. This small project will also help you develop your soldering skills and expose you to the circuit and PCB development process. Learning how to safely design and build dedicated power supplies will bring life to your current and future electronic projects.

A Problem named Covid

In addition to the pandemic, Covid created an instant demand for online at home learning. Suddenly electronics students everywhere lost access to Laboratory Bench Test Equipment and the direct supervision of a Lab Instructor. Modestly priced oscilloscopes, function generators, and laboratory DC power supplies exist and I encourage advanced students to acquire these tools as they can. However, the introductory student will not likely have access to these type of tools at home and for them the Breadboard PS along with a DMM will provide a, affordable and safe, at home circuit prototyping opportunity.

RMS & Peak Voltage

The wall outlet at your house is 120-volt rms. 120vrms is dangerous. The protection circuit breaker which acts like a safety switch, turning off the voltage, has the maximum current rating of 15 amps to 20 amps. Death can occur from as low as 100mA or 0.1A. The circuit breaker at your house is not designed to save your life from electrical shock, it is designed to prevent the wiring in your home from overheating heating and starting a fire. Do not mess 120v wall power it can kill you!

Figure 4. 120vrms waveform on oscilloscope

Wall voltage known as AC (Alternating Current) voltage. Most of what we do in electronics is DC (Direct Current). Examples of DC devices include: your computer, cell phone, Arduino, Raspberry Pi, and all things powered by batteries. The motors in your RC car are DC. However, some motors are designed to operate using single phase AC or industrial three phase AC voltage. Because our wall power is AC and our electronic circuits are mostly DC we will need to convert the alternating wall current to direct current. The process of converting AC to DC is called Rectification.

Figure 4 shows a captured image of 120vRMS wall AC. Observe that the Oscilloscope displays the image in alternating peak to peak voltage not RMS voltage. To convert peak to peak voltage to RMS the following formula can be used; Vrms= Vp2 OR Vp0.707 . Vp is peak voltage and is equal to Vpp (peak to peak voltage) divided by two (Vp=Vpp2 ). An Oscilloscope displays voltage along the Y axis and time along the X axis. In figure 5 note that channel 1 is set to 50 volts per division and have approximately six major divisions and two half divisions equaling 7 major divisions peak to peak. Now if we multiply 7div×50v/div we get a peak to peak voltage of 350vpp. Dividing 350vpp by 2 gives us a peak voltage of 175vp. Dividing 175vp by 2  gives an RMS voltage of 123.74vRMS≈120vRMS . Peak voltage is important to understand when we are using an oscilloscope and calculating rectification circuits. RMS is used for determining AC power. For example, the power calculations for an incandescent light bulb, audio amplifier, and ac motor would be calculated by multiplying the RMS voltage by the RMS current.

Voltage Rectification

Figure 5. Basic unregulated half-wave rectification

Voltage rectification is the process of changing AC (alternating current) to DC (direct current). Knowing that 100mA’s of current can be lethal and that the general skin resistance of a human is between 10KΩ & 1KΩ (when wet), we can use Ohm’s Law, where Current=VoltageResistance , to determine a relatively safe voltage. If we convert 120vrms to peak we get vp=120V×2 or vp=169.71vp.  Now assuming we were able to rectify the 169.71volts we now use the human resistance of 1KΩ to determine the current potential, i amps=169.71v1000Ω, i=169.71mA. This current is potentially lethal to a human. If 100mA is our threshold for lethal current we can calculate the max voltage by Vmax=1KΩ×100mA, Vmax=100v. Now we know that 100v and above are lethal voltages for humans and less than 100v should be less than lethal. Figure 5 shows a basic unregulated half-wave voltage rectifier, transformer T1 is used for isolation and safety. Notice that T1 has a 4:1 turns ratio. T1 is a step-down transformer meaning that the primary side (pins 1 & 2) have four

Figure 6. transformer secondary voltage waveform

times more turns than the secondary side (pins 3 & 4). A step-down transformers secondary voltage will be divided by the turns’ ratio. For this example, vSecondary=Vprimary4=169.71vp4=42.43vp. Figure 6 shows the secondary voltage waveform from T1 pin 3 to pin 4.  The voltage has been stepped down to around 42vp. The waveform however, is still AC. Notice the frequency for both fig 4 and fig 6. Frequency is measured in hertz (Hz) you will see that both the primary and the secondary are measuring at 60Hz which is the standard frequency for AC power in the United States.

Figure 7. diode anode and cathode

The diode D1 will achieve rectification by acting like a switch and allowing current to flow in only one direction.  When the anode is positive with respect to the cathode, current will flow. When the anode is negative with respect to the cathode the diode will act like an open switch and no current will flow. The output waveform will now appear to have the negative peaks clipped off (See fig. 8). The current is flowing in one direction and basic half-wave rectification has been achieved.  Notice the

Figure 8. half-wave rectification

waveform frequency is still 60Hz for half-wave rectification.

Capacitive Filtering

By adding a filtering capacitor, we can smooth the ripple waveform to make it more like a smooth DC. Figure 9 has added a filter capacitor C1 and load resistor R1. The capacitor will charge immediately when the diode D1 is forward biased. When the diode becomes reverse biased the capacitor will discharge through the load R1. With a large enough capacitance value and load resistor the output will be a smooth DC. We can test this by using a 10uF capacitor with no load, a 10KΩ load, and a 1KΩ load to show the varying degrees of ripple (see figures 10, 11, & 12).

Figure 9. basic unregulated rectifier with filtering

Calculating DC Voltage

When there is ripple present on the waveform, VDC can be thought of as the average waveform voltage. VDC can be calculated by adding the maximum positive peak voltage to the minimum peak voltage and then dividing the sum by two.

Example Calculate VDC for figure 12:

VDC=Vmax+Vmin2

Vmax=2.1 div×20vdiv=42v

Vmin=0.5 div ×20vdiv=10v. Now find VDC.

VDC1KΩ=Vmax+Vmin2= 42v+10v2=26V.

Un-Regulated

Figure 10. no load

Figure 11. 10K ohm load

Figure 12. 1K ohm load

Observe that as the load is increased (Ohm’s law I=VR, current or “load” increases as resistance is decreased), the voltage ripple waveform increases and as the ripple increases the effective DC voltage is decreased. This is an example of a rectifier circuit that is Un-Regulated. An un-regulated power supply’s DC voltage will fall as the circuit load current is increased.

Voltage Regulation

Figure 13. beta for a 2n3904 transistor

Figure 14. basic voltage regulation circuit

For many circuits having an unregulated power supply is a problem. Voltage regulation is required for any circuit that requires smooth consistent DC voltage across varying loads or current demands. Figure 13 shows a common collector amplifier which can be used to provide basic voltage regulation. To calculate the needed resistors R1 thru R4, an output voltage must be determined. Next, determine the maximum current need for your circuit. For this example, we will use 5V with a maximum current need of 5mA.  Calculate the base current (pin 2) of the transistor by dividing the 5mA emitter current (pin 2) by the minimum beta (aka hfe) per the data sheet. For the 2n3904. The minimum hfe for the 2n3904 with a collector current near 5mA will be will be between 80 and 100, see figure 14.

- IB≈5mA90≈55.556μA

With IB known, make IR2 a minimum of 10 times larger.

- IR2≥(10×IB)≥(10×55.556μA)≥555.556μA

Use Kirchhoff’s Voltage Law to calculate VR2.

- VR2=VR4+VBE=5V+0.7V=5.7V

Use Ohm’s Law to calculate R2

- R2=VR2IR2=5.7Vmin555.556μA=10.26KΩ

- Round down to next standard value, R2=10KΩ

Recalculate IR2 using standard value.

- IR2=VR2R2=5.7V10KΩ=570μA

Calculate IR1 using Kirchhoff’s Current Law.

- IR1=IB+IR2=55.556μA+570μA=625.556μA

Calculate VR1 using Kirchhoff’s Current Law.

- VR1= VCC-VR2=40V-5.7V=34.3V

Calculate R1 using Ohm’s Law.

- R1=VR1IR1=34.3V625.556μA=54.831KΩ

R3 is used to help split the power from the transistor. Power for the transistor is calculated using the max current multiplied by VCE.

- PQ1=ICmax×VCE

  - ICmax≈5mA

  - VCEmax=VCC-VR4=40V-5V=35V

  - Now make VCE= VCEmax2=352=17.5V

  - PQ1=ICmax×VCE=5mA×17.5V=87.5mW

Calculate VR3 using Kirchhoff’s Voltage Law.

- VR3=VCC-VCE-VR4=40V-17.5V-5V=17.5V

Calculate IR3.

- IR3≈5mA

Calculate R3 using Ohm’s Law.

- R3=VR3IR3=17.5V5mA=3.5KΩ

Figure 15. basic rectification circuit with basic voltage regulation

Figure 16. basic regulation circuit with standard values.

Figure 15 displays the regulation circuit with standard values. Figure 16 shows the entire rectification circuit with the voltage regulation added. Notice the second 10uF capacitor C2 at the base of the transistor. This is used to keep the voltage and current at the base of the transistor extra stable.  Figure 16 transistor orientation is also drawn in the more typical manner for a power supply.

Figure 17. CH2 regulated Vout, 1K ohm load

Figure 188. CH2 regulated Vout, 10K ohm load

Figure 19. CH2 regulated Vout, no load

Voltage Regulation Measured Output Waveforms

Observe Figure 17, 18, & 19. CH1 is the rectified input voltage across capacitor C1. CH2 is the regulated output which maintains a consistent DC voltage with no ripple from no load to a 1KΩ load.  If you look closely you can see a very small and tolerable shift in the DC level (approximately 0.5V) from no load to 1k. Now let’s practice what we have learned by designing basic 5VDC regulator using a 9VDC battery.

Your First Power Supply Project

We learned previously that the human body skin resistance is around 1KΩ when wet and 10KΩ when dry. We also learned that a current of 100mA is potentially lethal. If we use a 9VDC battery as our voltage source, what is the maximum current through wet skin and minimum current through dry potential exposure (use Ohm’s Law)?

- Wet skin: Iwet=VR=9V1KΩ=9mA

- Dry skin: Idry=VR=9V10KΩ=900μA

Figure 20. 5V regulator using a 9V source

With dry skin you will not be able to feel or perceive the less than 1mA of current from a 9V battery. If wet however, you could feel a non-lethal shock. Additionally, remove all metal jewelry prior to working with electronics.

First Challenge: Calculate and find the resistor values for the circuit in Figure 20 to produce a regulated 5V output with a max current of 5mA, assume Q1 is a 2n3904 with a minimum beta of 90.

Solution for First Challenge

Find the maximum base current IB for Q1.

- IBmax≈IEmaxBetamin≈5mA90≈55.556μA

Make IR2 ≥ 10 x IBmax.

- IR2≥10×55.556μA≥555.556μA

Find VR2 using Kirchhoff’s Voltage Law.

- VR2=VBE+VRL=0.7V+5V=5.7V

Find approximate R2 value using Ohm’s Law.

- R2≤VR2IR2≤5.7V555.556μA≤10.26KΩ

Round R2 value down to a standard value.

- R2=10KΩ

Recalculate IR2 using Ohm’s Law.

- IR2=VR2R2=5.7V10KΩ=570μA

Find IR1 using Kirchhoff’s Current Law.

- IR1=IR2+IB=570μA+55.556μA=625.556μA

Find VR1 using Kirchhoff’s Voltage Law.

- VR1=VBT1-VR2=9V-5.7V=3.3V

Calculate R1 using Ohm’s Law.

- R1=VR1IR1=3.3V625.556μA=5.28KΩ

Determine R3. First ask yourself what the total power of Q1 is without R3 and if that power needs to be split?

- PQ1=ICmax×VCE

  - VCEmax= VBT1-VRL=9V-5V=4V

  - ICmax=5mA

- PQ1=ICmax×VCE=5mA×4V=20mW

Figure 21. Test Circuit - R1 Standard Value, Additional Filter Cap C2.

The absolute maximum power rating for a 2N3904 is 625mW.  Optimal power operation for the 2N3904 will be less than or equal to half the absolute maximum rating, approximately 300mW. Because we are well below the optimal power operation at 20mW, R3 is not required for this circuit.

Figure 21. was assembled and tested. Measured 5.04V with a 1KΩ load resistor and 5.44V with no load. This circuit works good for a designed current range and a specific input voltage, in this case 9VDC. What happens to this circuit if the output is shorted? The answer is the transistor will exceed its power specifications and destroy itself. This circuit does not have current limiting protection. Another question might be how can we deal with a range of input voltages?

L7805 Regulator

Figure 22. L7805 5V linear regulator circuit.

The L7805 is a 5V fixed linear regulator with internally limited current and temperature protection. The L7805 can accept input voltages from 6V to 35V. The L7805 is part of a L78 series of linear regulators. The L78 series can be purchased as 5V, 6V, 8V, 8.5V, 9V, 12V, 15V, 18V, and 24V regulators. One point to understand is linear regulators can only regulate down, the cannot regulate up meaning the minimum input voltage for a 12V regulator must be 13V or more. For the L7805 5V regulator the input voltage can be 6V to an absolute maximum of 35V.

https://www.osha.gov/sites/default/files/2019-04/Basic_Electricity_Materials.pdf

https://media.digikey.com/pdf/Data%20Sheets/ST%20Microelectronics%20PDFS/2N3904.pdf

https://www.st.com/content/ccc/resource/technical/document/datasheet/41/4f/b3/b0/12/d4/47/88/CD00000444.pdf/files/CD00000444.pdf/jcr:content/translations/en.CD00000444.pdf